- TBD LED Screen Bluetooth Protocol V 1.0 4

# 1. Introduction

## 1.1 Purpose of the document

- Organize the general functions

- Provide bluetooth interaction protocol for display devices for APP and hardware developers to use

## 1.2 Scope of application (optional)

developer

## 1.3 Agreement version description

| edition | compile | date | explain |
| --- | --- | --- | --- |
| V1.0 | ZSQ | 2024.8.27 | First version agreement |
| V1.01 | ZSQ | 2024.8.28 | The program ID query command was deleted, the text program generation command was modified, and other small adjustments were made |
| V1.02 | ZSQ | 2024.11.05 | The special effects encoding has been redefined, the time calibration command has been added, the format definition of time programs has been re-adjusted, the broadcast package filter name and Bluetooth name have been modified, and other small adjustments have been made |
| V1.03 | ZSQ | 2024.11.27 | The color definition adds the full color 565 and 332 data format description, adds the total number of built-in pictures read protocol, adds the setting modification verification password function, and adds new function identification instructions |
| V1.0 4 | ZSQ | 2024.1 2.25 | Add a program type of pure gif file, and add whether the definition of remote control annotation position is supported in the broadcast package |

# 2. Function sorting

The basic functions are as follows:

- Send music and microphone rhythm

- Send real-time doodle data

- Send text programs

- Send a single BMP program

- Send GIFs or video programs

- Send multimedia text programs

- Send built-in image programs

- Set the brightness.

- Set the switch screen

- Set the selected program

- Set the rotation Angle

- Correct screen time

3.BLE services and features

Server UUID: 0000a950-0000-1000-8000-00805f9b34fb

Write1 Characteristic UUID: 0000a951-0000-1000-8000-00805f9b34fb is used to send encrypted data within 20 bytes

Write2 Characteristic UUID: 0000a952-0000-1000-8000-00805f9b34fb for sending big data (text and images, animations)

Notify UUID: 0000a953-0000-1000-8000-00805f9b34fb

- 4.B LE broadcast packet configuration

| category of the product | Filter name | Screen size | Screen color type | Screen rotation Angle | Firmware version number | client id | product function | Reserved positions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 54 42 44 02 | 00 0C 00 30 | 01 | 00 | 00 01 | 00 00 | 00 XX | 00 00 00 |
| Used to distinguish between different categories of products. 01: represents a dot matrix screen, | Byte0-byte1 (used to represent the current companys code, which is fixed for different products and is used to filter other companies Bluetooth devices when scanning for Bluetooth) byte2-byte3 (used to indicate the current APP encoding, used to distinguish different APP devices when scanning Bluetooth) | Byte0~byte1: screen height byte2~byte3: screen width | 0 0: The monochrome screen is driven 01: The monochrome screen is driven from top to bottom and from left to right 02: Rainbow 03: Full color 1 (888) 04: Full color 2 (565) 05: Full color 1 (332) | 00:0° 01:90° 02:180° 03:270° | Used to determine whether a firmware upgrade is needed | Used to distinguish which customers product starts from 00 01, 00 00 represents the public domain | To distinguish the functions of different products, see Table 1 Product Function Support Subdivision Table for the last byte expanded to 8 bits | Reserve 3 bytes for later expansion |

| XX | 8th | No.7 | 6th place | Fifth | No.4 | Number 3 | No.2 | Number 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| function definition | Remote control function | Password functionality | Border function | spinfunction | Multimedia programs | G if file encoding | subregion | time |
| explain | 0 no, 1 yes | 0 no, 1 yes | 0 no, 1 yes | 0 no, 1 yes | 0 no, 1 yes | 0 no, 1 yes | 0 no, 1 yes | 0 no, 1 yes |

Table 1 Product Function Support Subdivision Table

- BLE broadcast device name

iledcolor-XXXX (Note: The suffix XXXX is the last 4 characters of the BLE MAC address, and the prefix is set according to customer requirements)

- color coding

| black and white | colorful | Full color 1 (888) | Full color 2 (565) | Full color 3 (332) |
| --- | --- | --- | --- | --- |
| From left to right, the high position is in front | 0BGR0BGR format | RGB | RGB | RGB |
| Each 8 points are represented by 1 byte, corresponding to 1 bright and 0 black | Each 2 points are represented by 1 byte, black 0 (0000), red 0 (000 1 ), green 2 (00 1 0), blue 3 (00 11 ), yellow 4 (0 1 00), purple 5 (0 1 0 1 ), cyan 6 (0 110 ), white 7 (0 111 ) | Each point is represented by 3 bytes, which represent the grayscale value of R(0-255), G(0-255) and B(0-255) respectively. | Each point is represented by 2 bytes, which represent the grayscale value of R(0-255) shifted left by 3, the grayscale value of G(0-255) shifted right by 2, and the grayscale value of B(0-255) shifted right by 3 respectively. | Each point is represented by one byte, which represents the grayscale value of R(0-255) shifted right by 5 bits, the grayscale value of G(0-255) shifted right by 5 bits, and the grayscale value of B(0-255) shifted right by 6 bits respectively. |
| For example, 0x80 means one bright point and seven black points | For example, 0x73, which means to light up 1 white dot and 1 blue dot | For example, 0 Xff, 0x00,0xff represent the grayscale value of one point color: red (0xff), green (0x00), blue (0x ff) | For example, 0 Xf 8, 0x 1f represents the grayscale value of one point color: red (0xff), green (0x 00), blue (0x ff) | For example, 0 XE3 represents a grayscale value of one point color: red (0xff), green (0x 00), blue (0x ff) |

- Unified format instructions for data communications

# (1) APP distribution:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 00 | 00 05 | Nbyte | 00 00 |

- 1 complete package of sending data = identification code + command code + data length + data + check code

- Identification code: a fixed value used to identify the hardware protocol version.

- Command code description:

| command code | function Introduction |
| --- | --- |
| 00 | Subcontractors send program data instructions |
| 01 | Subcontractor sends a notification instruction to complete the work |
| 02 | Delete all program content instructions |
| 03 | Add program (text, BMP, GIF, time, multimedia text, built-in images) to playlist command |
| 04 | Send music and microphone rhythm commands |
| 05 | Send a command to send real-time doodle data |
| 06 | Temporary program send command |
| 07 | Select program instructions |
| 08 | Update the program playlist with the complete command |
| 09 | Set the brightness of the LED screen |
| 0A | Set switch LED screen command |
| 0B | Set the LED screen rotation Angle command |
| 0C | Correct the LED screen time command |
| 0D | Get the total number of built-in GIF commands |
| 0E | Set password command |
| 0F | Verify password instructions |

- Data length: refers to the number of bytes occupied by the data body (excluding identification code, command code, data length and the number of bytes occupied).

- Check code: the check code is calculated by adding up the sum of the current package from the identification code to the previous bit of the check code and taking the last 16 bits of the sum.

- Data: refers to the actual data body used in the current function, which can include multiple parameters. For example, setting a fixed pattern includes pattern number, effect, speed and brightness, which are four parameters in total, occupying 4 bytes.

# (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 00 | 00 0 3 | 00 00 00... | 00 00 |

# Processing result: The hardware processing result is returned according to the specific command

# Data sub-division

Due to the data being too long and the limited RAM space of the device, it is not acceptable to transmit too much data at once, and to improve the reliability of transmission, the data to be sent needs to be fragmented (data type, command code, data length, checksum are not included in fragmentation).

Sending channel: Writ e2

Format of each data package:

| IC | command code | DL | wrap id | Current packet data length | Current package data | check code |
| --- | --- | --- | --- | --- | --- | --- |
| 54 | 0 0 | 00 00 | 00 00 00 00 | 00 00 | Nbyte | 00 00 |
|  |  |  | 0~0xFFFFFFFF Each time a new text or other data that needs to be packaged is sent, the package id starts at 0 and then increases by 1 sequentially | Value range: 0~512 The MTU is set to be large Except for the last packet of each data length, no 0 is required |  |  |

pay attention to:

- Each packet data needs to be accompanied by data type. The equivalent data sent is: identification code + command code + data length + packet id + current packet data length + current packet data + check code

- Each packet sent by the device is replied to, and the APP needs to process the reply message.

- The app can only start the next action (transmitting the next packet, retransmitting data, showing transmission failure, etc.) after receiving a reply

Response: The corresponding package id transmission result is composed as follows:

| IC | command code | DL | wrap id | data | check code |
| --- | --- | --- | --- | --- | --- |
| 54 | 0 0 | 00 0 5 | 00 00 00 00 | 01 | 00 00 |
|  |  |  |  | 01. Transmission successful 00. Transmission failed 02. Equipment abnormality 03. Data error, setup failure, such as data length exceeds the maximum value |  |

When transmitting APP, timeout should be considered. If no reply is received after sending each packet within 1s, it indicates that the transmission has timed out, and it needs to be retransmitted (restarted). If the retransmission fails more than 3 times, it indicates that the data setting fails.

- (1) After the APP sends all the data, the sending instruction tells the hardware that the data has been sent.

Sending channel: Writ e2

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 1 | 00 0 3 | 01 | 00 00 |
|  |  |  | 01, Send all the subpackage data |  |

(2) Hardware requirements:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 1 | 00 0 3 | 01 | 00 00 |
|  |  |  | 01. Transmission successful 00. Transmission failed 02. Equipment abnormality 03. Data is incorrect and the setup fails, such as data length exceeds the maximum value |  |

# 9. Program package description

(1) Point data for each frame

16x16 points

The principle of taking the value is: take the data from left to right, from top to bottom, and the high position is in front.

According to the hardware color, the corresponding point definition is adopted. The color coding definition is shown in the 4-color coding definition in the LED protocol description.

Monochrome sampling definition: Each point is represented by a color number (0~1), where 0 is off and 1 is on. Eight points form one byte, with the high bit first, such as the first row of 16 points in the above figure, which consists of a total of 2 bytes of data E8,00, as shown below:

E8,00,00,00,3E,00,22,00,22,00,22,00,3E,00,03,E0,02,20,02,20,03,E0,00,7C,00,44,00,44,00,44,00,7C

Seven-color point definition: Each point is represented by a color number (0~7). The extinction is 0, and two points form a byte with the high bit first, such as the first line of 16 points in the above figure, which consists of 8 bytes of data in total, which are 12,30,70,00,00,00,00,00

The complete values are shown below:

12,30,70,00,00,00,00,00,00,00,00,00,00,00,00,00,00,11,11,10,00,00,00,00,00,10,00,10,00,00,00,00,00,10,00,10,00,00,00,00,00,10,00,10,00,00,00,00,00,11,11,10,00,00,00,00,00,00,00,22,22,20,00,00,00,00,00,20,00,20,00,00,00,00,00,20,00,20,00,00,00,00,00,22,22,20,00,00,00,00,00,00,03,33,33,00,00,00,00,00,03,00,03,00,00,00,00,00,03,00,03,00,00,00,00,00,03,00,03,00,00,00,00,00,03,33,33,00

Full-color sampling definition: Each point is represented by color (0~255,0~255,0~255), with 1 point expressed using 3 bytes in the order of R color component, G color component, B color component. As shown in the first row of the above figure, the 16 points total 48 bytes of data. To better match the special effects of LEDs, the final data saved after data transmission needs to perform gamma correction conversion on the following raw data, with a gamma value of 2.5

ED,1C,24,00,FF,00,00,00,FF,00,00,00,FF,FF,FF,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00

The complete values are shown below:

ED,1C,24,00,FF,00,00,00,FF,00,00,00,FF,FF,FF,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,ED,1C,24,ED,1C,24,ED,1C,24,ED,1C,24,ED,1C,24,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,ED,1C,24,00,00,00,00,00,00,00,00,00,ED,1C,24,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,ED,1C,24,00,00,00,00,00,00,00,00,00,ED,1C,24,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,ED,1C,24,00,00,00,00,00,00,00,00,00,ED,1C,24,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,ED,1C,24,ED,1C,24,ED,1C,24,ED,1C,24,ED,1C,24,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,00,00,00,00,

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,FF,00,00,00,00,00,00,

(2) The package description of the whole program

| Program ID | Total number of partitions | obligate | The specific content storage start position of the first partition | The specific content storage start position of the second partition | The specific content storage start position of the third partition | The specific content storage start position of the 4th partition | The data is packed by the specific content in sequence from 1 to 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00 00 00 00 | 01 | 00 00 00 | 00 00 00 00 | 00 00 00 00 | 00 00 00 00 | 00 00 00 00 | Nbyte |
| The unique identification code of the whole program CRC32-C check code | Indicates the total number of partitions in a single program | obligate | Start at 0 | The first starting address + the total length of data packed in the first partition | The first starting address + the total length of the data packed in the second partition | The first starting address + the total length of data packed in the third partition |  |

(3) Specific program content packing instructions

| The X and Y coordinates of the starting point of the partition | Partition size (LED_X, LED_Y) | obligate | program type | Total frames | specially good effect | velocity | residence time | Circular border | luminance | Reserve value | Specific program frame dot matrix data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 00 00 00 | 00 00 00 00 | 00 00 00 | 00 | 00 00 | 00 | 00 | 00 | 00 | 00 | 00 00 00 | Nbyte |
| Facing the LED screen, the top left corner coordinate is defined as (0,0) and the bottom right corner is defined as (LED_X-1, LED_Y-1) | The length can only be an multiple of 8 | obligate | Text programs 1. BMP program 2. GIF programs 3. Time programs 4. Multimedia text programs 5. Built-in GIF program (only for screens with built-in GIF) 6. GIF file format program | The total number of screens for the program content | 0: Fixed, 1: Left shift, 2: Right shift, 3: Upward shift, 4: Downward shift, 5: Snowflake 6: Scroll, 7: Laser, Only text, BMP and multimedia text programs are valid, and other programs are supplemented with 0 | 0-100 Except for time and GIF programs that are invalid and have to be supplemented with 0, all other programs are valid | 0 ¦ 1 0 0 | 0: No need for a circular border 1. Circular border 1 2. Circular border 2 3. Circular border 3 4. Circular border 4 | 0 ¦ 1 0 0 | obligate | In addition to text, time, multimedia text and built-in picture programs, the generation of other programs is the same, taking point array data from one screen and storing it in sequence |

- The data points of the text program are explained

Text programs generate screen data sequentially based on selecting font sizes on the corresponding partition size pixels. Except for the left or right shift effects where the duration is 0, this is when a full screen content is generated according to the partition size, extracting screen data accordingly. If its less than a full screen, it needs to be fully supplemented. Otherwise, the data is calculated based on font size accumulation until the corresponding partition size is reached, generating screen data accordingly, and then one screen at a time until a full screen is formed. Note that no half-character should appear in one screen, and half should appear in the next screen. Single-screen data is generated according to the explanation of taking dot matrix data at 9.1 per frame.

- Text program point array data description (reserved)

| Total number of characters | Horizontal alignment | Vertical alignment | obligate | Text parameter area | Font bitmap data area |
| --- | --- | --- | --- | --- | --- |
| 00 | 00 | 00 | 00 00 | Nbyte | Nbyte |
| 1 to 255 | 00: Left-aligned 01: Center alignment 02: Right alignment | 00: Top alignment 01: Center alignment 02: Bottom alignment |  | See the explanation section | See the explanation section |

explain:

Mold selection method:

Data is taken from left to right and from top to bottom. Each bit is bright as 1 and off as 0. The high bit is in front, and 0 is added if the bit is not enough

A character data = character parameter (stored in the parameter area) + character sampling data (character sampling data area)

1. Text parameters include content

| The length of a single word | The height of a single character | color value | obligate | The character is stored at the beginning of the character pattern dot matrix data area |
| --- | --- | --- | --- | --- |
| 00 | 00 | 00 00 00 | 00 | 00 00 00 00 |
| 1 to 255 | 0 to 255 | 00 00 00: monochrome 00 00 01: The last one is seven colors 00 00 00: RGB full color See color code table for details |  |  |

2. Generation of font dot matrix data Reference 9.1 Point matrix data for each frame

Note: If the text parameters are the same and there is a repetition, there is no need to regenerate the font, you can directly use the previous generated starting address to mark it for easy use,

For example, a monochrome screen sends a 12x12 "good" character. Example of data for a single character:

0C 0C 00 00 0000 00 00 00 00 20 00 27 C0 20 40 F8 8049 00 49 00 49 00 8F E0 51 00 21 00 51 00 96 00

Single color screen sends 212x12 "good" characters, single character data example:

0C 0C 00 00 00 00 00 00 00 000C 0C 00 00 00 00 00 00 00 00 20 00 27 C0 20 40 F8 80 49 00 49 00 49 00 8F E0 51 00 21 00 51 00 96 00

- The data of the point array of the BMP program (single figure) are described

Data is generated according to the size of a corresponding partition in a figure, and data is generated according to the point matrix data description of 9.1 per frame on a single screen.

- The GIF program (motion picture) illustrates the data points of the array

GIF is split into individual frames and then combined with the corresponding partition size to generate data screen by screen Note: 2 bytes must be added at the end of each screen data These 2 bytes represent the duration of the current frame The single screen data is generated according to the 9.1 frame point matrix data specification.

- Time program data explanation

| Format of the date | Date color | time format | Time color | The format of the week | Sunday color | body size | obligate | Displays the size of Chinese characters | Displays the ASCII size of characters | Character dot matrix data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 00 | 00 | 00 | 00 | 00 | 0 0 | 00 00 00 | 00 00 | 00 00 | Nbyte |
| 00: Do not display 01:2024.08.20 02:08.20 03:2024-08-20 04:08-20 05:08/20/2024 06:08/20 07:Aug 20,2024 08:Aug 20 09:20/08/2024 0A:20/08 0B:20 Aug 2024 0C:20 Aug 0D: August 12,2024 0E: August 12 | Red 01 Green 02 03 Blue 04 Yellow Purple 05 06 Green 07 White | 00: Do not display 01:15:20:15 02:15:20 03:03:20:15 PM 04:03:20 PM 05:03:20:15 06:03:20 | Red 01 Green 02 03 Blue 04 Yellow Purple 05 06 Green 07 White | 00: Do not display 01: Tues. 02: Tuesday 03: II | Red 01 Green 02 03 Blue 04 Yellow Purple 05 06 Green 07 White | 00: indicates custom 01: Built-in fonts 1 02: Built-in fonts 2 03: Built-in font 3 04: Built-in font 4 . . 0N: Built-in font N | obligate | The length and height of individual characters in Chinese content in pixels | The length and height of a single character in ASCII content | Characters that need to be displayed will be added later |

Note: The date, time and day of week format cannot be 0x00 at the same time. Only when the font size bit is 0x00, the subsequent characters are Chinese size and display characters

ASCII size and character dot matrix data are required and effective,

- The data of the point set for multimedia text program (text with background)

| Background GIF total frame count | Background GIF playback duration | obligate | Total data on the text content | Total data for the background GIF content |
| --- | --- | --- | --- | --- |
| 00 | 00 | 00 00 00 | Nbyte | Nbyte |
|  | 0 ~ 100 | obligate | Refer to 1 for the explanation of the data points for the text program | Refer to GIF program for point array data explanation |

Note: This definition of the program is understood as the background GIF is independent, and the points with content in the text program are the current points of the text, and the points without content are

Show the corresponding point of the GIF background.

- Built-in picture selection instructions (this function is only available if the hardware has built-in pictures)

| obligate | The program ID of the image |
| --- | --- |
| 00 00 00 00 | 00 00 00 00 |
|  | 0 ~ 100 |

- The GIF file program is stored in the dot matrix data area from beginning to end in the complete GIF file format

## 10. Send data to hardware (Note: Each time you perform the function in the playlist, you need to refresh the entire content of the playlist to ensure that the program data of APP and hardware are synchronized)

## 1. Delete all program content instructions

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 3 | 00 03 | 00 | 00 00 |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 3 | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. The deletion was successful |  |

## 2. Add program (text, BMP, GIF, time, multimedia text) to playlist command

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | The total number of programs in the playlist | Program number | program ID | Total program data length | Playback count | cheek | obligate | check code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54 | 03 | 00 10 | 00 | 00 | 00 00 00 00 | 00 00 00 00 | 0 1 | 00 | 00 00 | 00 00 |
|  |  |  | 0 to 255 | From 0 to 255, the position of the program to be queried in the playlist is marked from top to bottom by 1 to N | Program ID: CRC32-C | The total number of bytes in the current packaged program | 1-100 | 00: No need 0 1. Circular border 1 0 2. Circular border 2 0 3. Circular border 3 0 4. Circular border 4 |  |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 3 | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Can send new data 2. Insufficient space 3. The program already exists and does not need to be updated 4 Data errors, such as data length exceeding the maximum value... |  |

### (3) According to the result of 2, if it is 1, new data can be sent, and the packed program data can be sent according to the subpackage

## 3. Send music and microphone rhythm commands

- Command transmitted by the APP to the hardware (sending channel: Write2)

| IC | command code | DL | Dancing state | data | check code |
| --- | --- | --- | --- | --- | --- |
| 54 | 0 4 | 00 00 | 00 | N byte | 00 00 |
|  |  |  | 00: Exit the rhythm display function 01: Enter the dynamic display function | ### The specific format is to be determined |  |

- The hardware does not need to be replied

## 4. Send a command to send real-time doodle data

## (1) Commands transmitted by APP to hardware (sending channel: Write 1)

| IC | command code | DL | Graffiti mode | data | check code |
| --- | --- | --- | --- | --- | --- |
| 54 | 0 5 | 00 09 | 00 | 00 00 00 00 00 00 | 00 00 |
|  |  |  | 1. Enter the doodle and clear the current display content 2. Enter the doodle without clearing the current display content 3. Exiting the doodle still displays the doodle content 4. Exit the doodle without saving data | byte0: Type (0. Normal drawing, 1. Horizontal mirror, 2. Vertical mirror, 3. Fill, 4. Erase) Byte1: indicates the current line number byte2: indicates the current column number Byte3-Byte5: See 4 color coding definition, and fill in the excess part with 0 |  |

- The hardware does not need to be replied to

### Note: To avoid losing data, when you lift your hand after drawing, you can send the whole picture of the current doodle (see packing instructions for the data generation format) to the hardware. Refer to BM P program for the sending method

## 5. Send temporary program instructions

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | program ID | Total program data length | cheek | obligate | check code |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 54 | 0 6 | 00 0D | 00 00 00 00 | 00 00 00 00 | 00 | 00 00 | 00 00 |
|  |  |  | The unique ID code of the program: CRC32-C | The total number of bytes in the current packaged program | 00: No need 0 1. Circular border 1 0 2. Circular border 2 0 3. Circular border 3 0 4. Circular border 4 |  |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 6 | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Transmission is successful 2. Insufficient space 3. Data does not need to be updated |  |

### (3) According to the result 2, if the transmission is successful, the packed program data can be sent according to the subpackage

## 6. Select program instructions

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | bit zone | Program number | data | check code |
| --- | --- | --- | --- | --- | --- | --- |
| 54 | 07 | 00 0 9 | 00 | 00 00 | 00 00 00 00 | 00 00 |
|  |  |  | 00: Specify the program 01: Randomly play from the playlist 02: Play in playlist order 0 3: Play the previous item in the playlist 0 4: Play the next item in the playlist | The number of the selected programs in the playlist is marked from top to bottom as 1 to N, and only when the flag =00 is valid | obligate |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 7 | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Selection is successful 2. The show does not exist |  |

## 7. Complete the update program playlist command

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e 1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 08 | 00 03 | 01 | 00 00 |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 8 | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Operation is successful |  |

## 8. Set the LED screen brightness command

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | brightness value | obligate | check code |
| --- | --- | --- | --- | --- | --- |
| 54 | 09 | 00 0 A | 0 1 | 00 | 00 00 |
|  |  |  | 01: Minimum brightness 02: Medium brightness 03: High brightness |  |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 9 | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Settings are successful |  |

## 9. Set switch LED screen command

(1) Commands transmitted by APP to hardware (sending channel: Writ e1)

| IC | command code | DL | data | obligate | check code |
| --- | --- | --- | --- | --- | --- |
| 54 | 0A | 00 0A | 00 | 00 00 00 00 00 00 00 00 | 00 00 |
|  |  |  | 00: Turn off the screen 01: Splash screen |  |  |

- Hardware response: none

## 10. Set the rotation Angle of LED screen (reserved)

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 B | 00 03 | 00 | 00 00 |
|  |  |  | 00:0 (360) degrees 0 1:90 degrees 0 2:180 degrees 0 3:270 degrees |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 B | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Settings are successful |  |

## 1. Correct the LED screen time

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 C | 00 0 9 | 00 0 0 0 0 0 0 00 00 00 | 00 00 |
|  |  |  | Two values at the end of 1 year 2. Month 3. Day 4. Time 5. Division 6. Seconds 7. Reserve |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 C | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Settings are successful |  |

## 1 2. Query the total number of built-in images (only products with built-in images are valid)

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 D | 00 0 3 | 00 | 00 00 |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 D | 00 03 | 00 | 00 00 |
|  |  |  | 00-FF indicates the total number of built-in gifs |  |

## 1 3. Set a password

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | FC | Old passwords | New password | check code |
| --- | --- | --- | --- | --- | --- | --- |
| 54 | 0E | 00 0 F | 00 | 00 00 00 00 00 00 | 00 00 00 00 00 00 | 00 00 |
|  |  |  | 00: Set password (only effective on hosts that have not enabled the password, the password is filled into the new password position, and 6 bytes of data need to be added to the old password position) 01: Modify the password (both the old password and the new password need to be filled in. The logic will first verify the old password and update the password after confirmation) 02: Delete the password (the old password needs to be filled in, the new password does not need to be filled in, the logic will first verify the old password, and then update the password) | The value of the 6-character password | The value of a 6-character password |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 4 | 0 E | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Set up successfully↔ 2. The modification failed because the old password was incorrect 3. Delete failed, the old password is not correct |  |

## 1 4. Verify the password

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 F | 00 0 8 | 00 00 00 00 00 00 | 00 00 |
|  |  |  | The value of the 6-character password |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 0 F | 00 03 | 00 | 00 00 |
|  |  |  | 0. Transmission failed 1. Verification is successful 2. Verification failed 3. Password function is not enabled |  |

## 1 5. Add a query command to determine whether the firmware supports new features

(1) Commands transmitted by the mobile phone to the hardware (sending channel: Writ e1)

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 10 | 00 08 | 00 | 00 00 |
|  |  |  | 00: Time function 01: Partition function 02: Rhythm function 03: Password function 04: GIF file storage function |  |

### (2) Hardware response:

| IC | command code | DL | data | check code |
| --- | --- | --- | --- | --- |
| 54 | 10 | 00 03 | 00 | 00 00 |
|  |  |  | 00: Not supported 01: Support |  |